# TEST REPORT

## SUMMARY

Description                "scenario_vlan" result report

Start Time                  2026-09-22T18:55:31.107542

| Index | Scenario | Time Stamp | Pass/Fail |
| --- | --- | --- | --- |
| 1 | Print Topology Image | 18:56:20 | 1 / 0 |
| 2 | Session Check | 18:56:20 | 6 / 0 |
| 3 | Check SONiC OS Version | 18:56:20 | 2 / 0 |
| 4 | Create Vlan | 18:56:31 | 10 / 0 |
| 5 | Add Vlan Member | 18:57:27 | 10 / 0 |
| 6 | Assign IP Address to Host | 18:58:23 | 8 / 0 |
| 7 | Test Network-link | 18:58:23 | 2 / 0 |
| 8 | Delete IP Address to Host | 18:58:32 | 8 / 0 |

## SESSIONS

| Name | Connection Type | Host Address | Port | Login User |
| --- | --- | --- | --- | --- |
| H-1 | ssh | 10.250.0.101 | 22 | ubuntu |
| H-2 | ssh | 10.250.0.102 | 22 | ubuntu |
| H-3 | ssh | 10.250.0.103 | 22 | ubuntu |
| H-4 | ssh | 10.250.0.104 | 22 | ubuntu |
| SW-1 | ssh | 10.250.0.10 | 22 | admin |
| SW-2 | ssh | 10.250.0.11 | 22 | admin |

## RESULT

| Index | Action | Session | Time Stamp | Pass/Fail |
| --- | --- | --- | --- | --- |
| 1 | print topo img | caller | 18:56:20 | Pass |
| Parameter | topo-L2-Vlan.jpg |  |  |  |
| Response |  |  |  |  |
| 2 | session check SW-1 | SW-1 | 18:56:20 | Pass |
| Parameter | None |  |  |  |
| Response | Linux SW-1 6.1.0-29-2-amd64 #1 SMP PREEMPT_DYNAMIC Debian 6.1.123-1 (2025-01-02) x86_64 You are on ____ ___ _ _ _ ____ / ___¦ / _ \¦ \ ¦ (_)/ ___¦ \___ \¦ ¦ ¦ ¦ \¦ ¦ ¦ ¦ ___) ¦ ¦_¦ ¦ ¦\ ¦ ¦ ¦___ ¦____/ \___/¦_¦ \_¦_¦\____¦ -- Software for Open Networking in the Cloud -- Unauthorized access and/or use are prohibited. All access and/or use are subject to monitoring. Help: https://sonic-net.github.io/SONiC/ Last login: Tue Sep 22 18:48:00 2026 from 10.250.0.1 admin@SW-1:~$ |  |  |  |
| 3 | session check SW-2 | SW-2 | 18:56:20 | Pass |
| Parameter | None |  |  |  |
| Response | Linux SW-2 6.1.0-29-2-amd64 #1 SMP PREEMPT_DYNAMIC Debian 6.1.123-1 (2025-01-02) x86_64 You are on ____ ___ _ _ _ ____ / ___¦ / _ \¦ \ ¦ (_)/ ___¦ \___ \¦ ¦ ¦ ¦ \¦ ¦ ¦ ¦ ___) ¦ ¦_¦ ¦ ¦\ ¦ ¦ ¦___ ¦____/ \___/¦_¦ \_¦_¦\____¦ -- Software for Open Networking in the Cloud -- Unauthorized access and/or use are prohibited. All access and/or use are subject to monitoring. Help: https://sonic-net.github.io/SONiC/ Last login: Tue Sep 22 18:48:02 2026 from 10.250.0.1 admin@SW-2:~$ |  |  |  |
| 4 | session check H-1 | H-1 | 18:56:20 | Pass |
| Parameter | None |  |  |  |
| Response | Welcome to Ubuntu 20.04.6 LTS (GNU/Linux 5.4.0-215-generic x86_64) * Documentation: https://help.ubuntu.com * Management: https://landscape.canonical.com * Support: https://ubuntu.com/pro System information as of Tue Sep 22 09:56:06 UTC 2026 System load: 0.04 Processes: 145 Usage of /: 9.0% of 19.20GB Users logged in: 0 Memory usage: 6% IPv4 address for Eth1: 10.0.100.10 Swap usage: 0% Expanded Security Maintenance for Applications is not enabled. 0 updates can be applied immediately. Enable ESM Apps to receive additional future security updates. See https://ubuntu.com/esm or run: sudo pro status The list of available updates is more than a week old. To check for new updates run: sudo apt update New release '22.04.5 LTS' available. Run 'do-release-upgrade' to upgrade to it. *** System restart required *** Last login: Tue Sep 22 09:48:05 2026 from 10.250.0.1 ubuntu@H-1:~$ |  |  |  |
| 5 | session check H-2 | H-2 | 18:56:20 | Pass |
| Parameter | None |  |  |  |
| Response | Welcome to Ubuntu 20.04.6 LTS (GNU/Linux 5.4.0-215-generic x86_64) * Documentation: https://help.ubuntu.com * Management: https://landscape.canonical.com * Support: https://ubuntu.com/pro System information as of Tue Sep 22 09:56:08 UTC 2026 System load: 0.04 Processes: 140 Usage of /: 9.0% of 19.20GB Users logged in: 0 Memory usage: 6% IPv4 address for Eth1: 10.0.100.11 Swap usage: 0% Expanded Security Maintenance for Applications is not enabled. 0 updates can be applied immediately. Enable ESM Apps to receive additional future security updates. See https://ubuntu.com/esm or run: sudo pro status The list of available updates is more than a week old. To check for new updates run: sudo apt update New release '22.04.5 LTS' available. Run 'do-release-upgrade' to upgrade to it. *** System restart required *** Last login: Tue Sep 22 09:48:07 2026 from 10.250.0.1 ubuntu@H-2:~$ |  |  |  |
| 6 | session check H-3 | H-3 | 18:56:20 | Pass |
| Parameter | None |  |  |  |
| Response | Welcome to Ubuntu 20.04.6 LTS (GNU/Linux 5.4.0-215-generic x86_64) * Documentation: https://help.ubuntu.com * Management: https://landscape.canonical.com * Support: https://ubuntu.com/pro System information as of Tue Sep 22 09:56:10 UTC 2026 System load: 0.0 Processes: 144 Usage of /: 9.0% of 19.20GB Users logged in: 0 Memory usage: 6% IPv4 address for Eth1: 10.0.100.12 Swap usage: 0% IPv4 address for Eth1: 10.0.200.10 Expanded Security Maintenance for Applications is not enabled. 0 updates can be applied immediately. Enable ESM Apps to receive additional future security updates. See https://ubuntu.com/esm or run: sudo pro status The list of available updates is more than a week old. To check for new updates run: sudo apt update New release '22.04.5 LTS' available. Run 'do-release-upgrade' to upgrade to it. *** System restart required *** Last login: Tue Sep 22 09:48:09 2026 from 10.250.0.1 ubuntu@H-3:~$ |  |  |  |
| 7 | session check H-4 | H-4 | 18:56:20 | Pass |
| Parameter | None |  |  |  |
| Response | Welcome to Ubuntu 20.04.6 LTS (GNU/Linux 5.4.0-215-generic x86_64) * Documentation: https://help.ubuntu.com * Management: https://landscape.canonical.com * Support: https://ubuntu.com/pro System information as of Tue Sep 22 09:56:12 UTC 2026 System load: 0.0 Processes: 144 Usage of /: 9.0% of 19.20GB Users logged in: 0 Memory usage: 6% IPv4 address for Eth1: 10.0.100.13 Swap usage: 0% IPv4 address for Eth1: 10.0.200.11 Expanded Security Maintenance for Applications is not enabled. 0 updates can be applied immediately. Enable ESM Apps to receive additional future security updates. See https://ubuntu.com/esm or run: sudo pro status The list of available updates is more than a week old. To check for new updates run: sudo apt update New release '22.04.5 LTS' available. Run 'do-release-upgrade' to upgrade to it. *** System restart required *** Last login: Tue Sep 22 09:48:12 2026 from 10.250.0.1 ubuntu@H-4:~$ |  |  |  |
| 8 | Check SONiC OS Version | SW-1 | 18:56:20 | Pass |
| Parameter | ~$ show version |  |  |  |
| Response | SONiC Software Version: SONiC.202505.926279-b7206d85a SONiC OS Version: 12 Distribution: Debian 12.11 Kernel: 6.1.0-29-2-amd64 Build commit: b7206d85a Build date: Tue Aug 19 14:16:31 UTC 2025 Built by: azureuser@d674752fc000000 Platform: x86_64-kvm_x86_64-r0 HwSKU: Etri-SONiC ASIC: vs ASIC Count: 1 Serial Number: N/A Model Number: N/A Hardware Revision: N/A Uptime: 18:56:20 up 26 min, 1 user, load average: 0.61, 1.28, 1.07 Date: Tue 22 Sep 2026 18:56:20 Docker images: REPOSITORY TAG IMAGE ID SIZE docker-macsec 202505.926279-b7206d85a eb3fac1f1e14 347MB docker-macsec latest eb3fac1f1e14 347MB docker-orchagent 202505.926279-b7206d85a 1d3a6e9b7605 356MB docker-orchagent latest 1d3a6e9b7605 356MB docker-sflow 202505.926279-b7206d85a f7c21d48ff02 345MB docker-sflow latest f7c21d48ff02 345MB docker-teamd 202505.926279-b7206d85a 219394382f06 344MB docker-teamd latest 219394382f06 344MB docker-fpm-frr 202505.926279-b7206d85a 231b5d766152 393MB docker-fpm-frr latest 231b5d766152 393MB docker-nat 202505.926279-b7206d85a 553569c2ed1a 347MB docker-nat latest 553569c2ed1a 347MB docker-dash-ha 202505.926279-b7206d85a eadcbe8a318f 357MB docker-dash-ha latest eadcbe8a318f 357MB docker-syncd-vs 202505.926279-b7206d85a d6f77d7d5c73 2.18GB docker-syncd-vs latest d6f77d7d5c73 2.18GB docker-gbsyncd-vs 202505.926279-b7206d85a e1b2e95f057b 2.18GB docker-gbsyncd-vs latest e1b2e95f057b 2.18GB docker-sonic-bmp 202505.926279-b7206d85a 773e76990a92 315MB docker-sonic-bmp latest 773e76990a92 315MB docker-dhcp-relay 202505.926279-b7206d85a b82ba73b313a 323MB docker-dhcp-relay latest b82ba73b313a 323MB docker-platform-monitor 202505.926279-b7206d85a dff6c6e2ea25 447MB docker-platform-monitor latest dff6c6e2ea25 447MB docker-snmp 202505.926279-b7206d85a f813952ac4d7 358MB docker-snmp latest f813952ac4d7 358MB docker-router-advertiser 202505.926279-b7206d85a e8cbc5b10fff 314MB docker-router-advertiser latest e8cbc5b10fff 314MB docker-mux 202505.926279-b7206d85a a6ccc3ca8b81 366MB docker-mux latest a6ccc3ca8b81 366MB docker-gnmi-watchdog 202505.926279-b7206d85a 9871849025e5 314MB docker-gnmi-watchdog latest 9871849025e5 314MB docker-lldp 202505.926279-b7206d85a ee7a4d514875 360MB docker-lldp latest ee7a4d514875 360MB docker-sonic-gnmi 202505.926279-b7206d85a 9f8ed5aefa5d 427MB docker-sonic-gnmi latest 9f8ed5aefa5d 427MB docker-bmp-watchdog 202505.926279-b7206d85a 62eb09ee3d7e 314MB docker-bmp-watchdog latest 62eb09ee3d7e 314MB docker-eventd 202505.926279-b7206d85a b5707e85d1ec 314MB docker-eventd latest b5707e85d1ec 314MB docker-database 202505.926279-b7206d85a 6d85154e4f52 321MB docker-database latest 6d85154e4f52 321MB docker-auditd-watchdog 202505.926279-b7206d85a 92489e94897e 314MB docker-auditd-watchdog latest 92489e94897e 314MB docker-auditd 202505.926279-b7206d85a 5e7f418566b0 314MB docker-auditd latest 5e7f418566b0 314MB docker-dash-engine 202505.926279-b7206d85a 9b719283bb99 697MB docker-dash-engine latest 9b719283bb99 697MB docker-sonic-mgmt-framework 202505.926279-b7206d85a 347d1879e56e 403MB docker-sonic-mgmt-framework latest 347d1879e56e 403MB docker-sysmgr 202505.926279-b7206d85a acdc8191fd32 326MB docker-sysmgr latest acdc8191fd32 326MB |  |  |  |
| Judgement | Success inter-conditional operator : or contain string: SONiC OS Version: 12 contain string: SONiC Software Version: SONiC.master.775830-a7eb965c8 not contain string: Error: |  |  |  |
| 9 | Check SONiC OS Version | SW-2 | 18:56:25 | Pass |
| Parameter | ~$ show version |  |  |  |
| Response | SONiC Software Version: SONiC.202505.926279-b7206d85a SONiC OS Version: 12 Distribution: Debian 12.11 Kernel: 6.1.0-29-2-amd64 Build commit: b7206d85a Build date: Tue Aug 19 14:16:31 UTC 2025 Built by: azureuser@d674752fc000000 Platform: x86_64-kvm_x86_64-r0 HwSKU: Etri-SONiC ASIC: vs ASIC Count: 1 Serial Number: N/A Model Number: N/A Hardware Revision: N/A Uptime: 18:56:25 up 26 min, 1 user, load average: 1.50, 1.63, 1.19 Date: Tue 22 Sep 2026 18:56:25 Docker images: REPOSITORY TAG IMAGE ID SIZE docker-macsec 202505.926279-b7206d85a eb3fac1f1e14 347MB docker-macsec latest eb3fac1f1e14 347MB docker-orchagent 202505.926279-b7206d85a 1d3a6e9b7605 356MB docker-orchagent latest 1d3a6e9b7605 356MB docker-sflow 202505.926279-b7206d85a f7c21d48ff02 345MB docker-sflow latest f7c21d48ff02 345MB docker-teamd 202505.926279-b7206d85a 219394382f06 344MB docker-teamd latest 219394382f06 344MB docker-fpm-frr 202505.926279-b7206d85a 231b5d766152 393MB docker-fpm-frr latest 231b5d766152 393MB docker-nat 202505.926279-b7206d85a 553569c2ed1a 347MB docker-nat latest 553569c2ed1a 347MB docker-dash-ha 202505.926279-b7206d85a eadcbe8a318f 357MB docker-dash-ha latest eadcbe8a318f 357MB docker-gbsyncd-vs 202505.926279-b7206d85a e1b2e95f057b 2.18GB docker-gbsyncd-vs latest e1b2e95f057b 2.18GB docker-syncd-vs 202505.926279-b7206d85a d6f77d7d5c73 2.18GB docker-syncd-vs latest d6f77d7d5c73 2.18GB docker-sonic-bmp 202505.926279-b7206d85a 773e76990a92 315MB docker-sonic-bmp latest 773e76990a92 315MB docker-dhcp-relay 202505.926279-b7206d85a b82ba73b313a 323MB docker-dhcp-relay latest b82ba73b313a 323MB docker-platform-monitor 202505.926279-b7206d85a dff6c6e2ea25 447MB docker-platform-monitor latest dff6c6e2ea25 447MB docker-snmp 202505.926279-b7206d85a f813952ac4d7 358MB docker-snmp latest f813952ac4d7 358MB docker-router-advertiser 202505.926279-b7206d85a e8cbc5b10fff 314MB docker-router-advertiser latest e8cbc5b10fff 314MB docker-mux 202505.926279-b7206d85a a6ccc3ca8b81 366MB docker-mux latest a6ccc3ca8b81 366MB docker-gnmi-watchdog 202505.926279-b7206d85a 9871849025e5 314MB docker-gnmi-watchdog latest 9871849025e5 314MB docker-lldp 202505.926279-b7206d85a ee7a4d514875 360MB docker-lldp latest ee7a4d514875 360MB docker-sonic-gnmi 202505.926279-b7206d85a 9f8ed5aefa5d 427MB docker-sonic-gnmi latest 9f8ed5aefa5d 427MB docker-bmp-watchdog 202505.926279-b7206d85a 62eb09ee3d7e 314MB docker-bmp-watchdog latest 62eb09ee3d7e 314MB docker-eventd 202505.926279-b7206d85a b5707e85d1ec 314MB docker-eventd latest b5707e85d1ec 314MB docker-database 202505.926279-b7206d85a 6d85154e4f52 321MB docker-database latest 6d85154e4f52 321MB docker-auditd-watchdog 202505.926279-b7206d85a 92489e94897e 314MB docker-auditd-watchdog latest 92489e94897e 314MB docker-auditd 202505.926279-b7206d85a 5e7f418566b0 314MB docker-auditd latest 5e7f418566b0 314MB docker-dash-engine 202505.926279-b7206d85a 9b719283bb99 697MB docker-dash-engine latest 9b719283bb99 697MB docker-sonic-mgmt-framework 202505.926279-b7206d85a 347d1879e56e 403MB docker-sonic-mgmt-framework latest 347d1879e56e 403MB docker-sysmgr 202505.926279-b7206d85a acdc8191fd32 326MB docker-sysmgr latest acdc8191fd32 326MB |  |  |  |
| Judgement | Success inter-conditional operator : or contain string: SONiC OS Version: 12 contain string: SONiC Software Version: SONiC.master.775830-a7eb965c8 not contain string: Error: |  |  |  |
| 10 | Check VLAN exist on SW-1 | SW-1 | 18:56:31 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- -------- ------ |  |  |  |
| Judgement | Success inter-conditional operator : and not contain string: Error: |  |  |  |
| 11 | Check VLAN exist on SW-2 | SW-2 | 18:56:37 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- -------- ------ |  |  |  |
| Judgement | Success inter-conditional operator : and not contain string: Error: |  |  |  |
| 12 | Create VLAN 10 on SW-1 | SW-1 | 18:56:42 | Pass |
| Parameter | ~$ sudo config vlan add 10 |  |  |  |
| Response | Success |  |  |  |
| 13 | Create VLAN 10 on SW-1 | SW-1 | 18:56:48 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- -------- ------ Vlan10 10 |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: Vlan10 10 not contain string: Error: |  |  |  |
| 14 | Create VLAN 10 on SW-2 | SW-2 | 18:56:53 | Pass |
| Parameter | ~$ sudo config vlan add 10 |  |  |  |
| Response | Success |  |  |  |
| 15 | Create VLAN 10 on SW-2 | SW-2 | 18:56:59 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- -------- ------ Vlan10 10 |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: Vlan10 10 not contain string: Error: |  |  |  |
| 16 | Create VLAN 20 on SW-1 | SW-1 | 18:57:04 | Pass |
| Parameter | ~$ sudo config vlan add 20 |  |  |  |
| Response | Success |  |  |  |
| 17 | Create VLAN 20 on SW-1 | SW-1 | 18:57:10 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- -------- ------ Vlan10 10 Vlan20 20 |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: Vlan20 20 not contain string: Error: |  |  |  |
| 18 | Create VLAN 20 on SW-2 | SW-2 | 18:57:16 | Pass |
| Parameter | ~$ sudo config vlan add 20 |  |  |  |
| Response | Success |  |  |  |
| 19 | Create VLAN 20 on SW-2 | SW-2 | 18:57:21 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- -------- ------ Vlan10 10 Vlan20 20 |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: Vlan20 20 not contain string: Error: |  |  |  |
| 20 | Add VLAN 10 & VLAN 20 member on SW-1 | SW-1 | 18:57:27 | Pass |
| Parameter | ~$ sudo config vlan member add 10 Ethernet8 |  |  |  |
| Response | Success |  |  |  |
| 21 | Add VLAN 10 & VLAN 20 member on SW-1 | SW-1 | 18:57:32 | Pass |
| Parameter | ~$ sudo config vlan member add 20 Ethernet8 |  |  |  |
| Response | Success |  |  |  |
| 22 | Add VLAN 10 & VLAN 20 member on SW-1 | SW-1 | 18:57:38 | Pass |
| Parameter | ~$ sudo config vlan member add 10 -u Ethernet16 |  |  |  |
| Response | Success |  |  |  |
| 23 | Add VLAN 10 & VLAN 20 member on SW-1 | SW-1 | 18:57:44 | Pass |
| Parameter | ~$ sudo config vlan member add 20 -u Ethernet20 |  |  |  |
| Response | Success |  |  |  |
| 24 | Add VLAN 10 & VLAN 20 member on SW-1 | SW-1 | 18:57:49 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- ---------- -------- Vlan10 10 Ethernet8 tagged Vlan10 10 Ethernet16 untagged Vlan20 20 Ethernet8 tagged Vlan20 20 Ethernet20 untagged |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: Vlan10 10 Ethernet8 tagged contain string: Vlan10 10 Ethernet16 untagged contain string: Vlan20 20 Ethernet8 tagged contain string: Vlan20 20 Ethernet20 untagged not contain string: Error: |  |  |  |
| 25 | Add VLAN 10 & VLAN 20 member on SW-2 | SW-2 | 18:57:55 | Pass |
| Parameter | ~$ sudo config vlan member add 10 Ethernet4 |  |  |  |
| Response | Success |  |  |  |
| 26 | Add VLAN 10 & VLAN 20 member on SW-2 | SW-2 | 18:58:00 | Pass |
| Parameter | ~$ sudo config vlan member add 20 Ethernet4 |  |  |  |
| Response | Success |  |  |  |
| 27 | Add VLAN 10 & VLAN 20 member on SW-2 | SW-2 | 18:58:06 | Pass |
| Parameter | ~$ sudo config vlan member add 10 -u Ethernet12 |  |  |  |
| Response | Success |  |  |  |
| 28 | Add VLAN 10 & VLAN 20 member on SW-2 | SW-2 | 18:58:12 | Pass |
| Parameter | ~$ sudo config vlan member add 20 -u Ethernet16 |  |  |  |
| Response | Success |  |  |  |
| 29 | Add VLAN 10 & VLAN 20 member on SW-2 | SW-2 | 18:58:17 | Pass |
| Parameter | ~$ show vlan config |  |  |  |
| Response | Name VID Member Mode ------ ----- ---------- -------- Vlan10 10 Ethernet4 tagged Vlan10 10 Ethernet12 untagged Vlan20 20 Ethernet4 tagged Vlan20 20 Ethernet16 untagged |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: Vlan10 10 Ethernet4 tagged contain string: Vlan10 10 Ethernet12 untagged contain string: Vlan20 20 Ethernet4 tagged contain string: Vlan20 20 Ethernet16 untagged not contain string: Error: |  |  |  |
| 30 | Set Eth1 IPv4 Address on H-1 | H-1 | 18:58:23 | Pass |
| Parameter | ~$ sudo ip address add 10.0.100.10/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 31 | Set Eth1 IPv4 Address on H-1 | H-1 | 18:58:23 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:01 brd ff:ff:ff:ff:ff:ff inet 10.0.100.10/24 brd 10.0.100.255 scope global Eth1 valid_lft forever preferred_lft forever inet6 fe80::5054:ff:fe10:201/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: inet 10.0.100.10/24 not contain string: DOWN |  |  |  |
| 32 | Set Eth1 IPv4 Address on H-2 | H-2 | 18:58:23 | Pass |
| Parameter | ~$ sudo ip address add 10.0.100.11/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 33 | Set Eth1 IPv4 Address on H-2 | H-2 | 18:58:23 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:02 brd ff:ff:ff:ff:ff:ff inet 10.0.100.11/24 brd 10.0.100.255 scope global Eth1 valid_lft forever preferred_lft forever inet6 fe80::5054:ff:fe10:202/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: inet 10.0.100.11/24 not contain string: DOWN |  |  |  |
| 34 | Set Eth1 IPv4 Address on H-3 | H-3 | 18:58:23 | Pass |
| Parameter | ~$ sudo ip address add 10.0.200.10/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 35 | Set Eth1 IPv4 Address on H-3 | H-3 | 18:58:23 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:03 brd ff:ff:ff:ff:ff:ff inet 10.0.100.12/24 brd 10.0.100.255 scope global Eth1 valid_lft forever preferred_lft forever inet 10.0.200.10/24 scope global Eth1 valid_lft forever preferred_lft forever inet6 fe80::5054:ff:fe10:203/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: inet 10.0.200.10/24 not contain string: DOWN |  |  |  |
| 36 | Set Eth1 IPv4 Address on H-4 | H-4 | 18:58:23 | Pass |
| Parameter | ~$ sudo ip address add 10.0.200.11/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 37 | Set Eth1 IPv4 Address on H-4 | H-4 | 18:58:23 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:04 brd ff:ff:ff:ff:ff:ff inet 10.0.100.13/24 brd 10.0.100.255 scope global Eth1 valid_lft forever preferred_lft forever inet 10.0.200.11/24 scope global Eth1 valid_lft forever preferred_lft forever inet6 fe80::5054:ff:fe10:204/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: inet 10.0.200.11/24 not contain string: DOWN |  |  |  |
| 38 | Ping from H-1 to H-2 | H-1 | 18:58:23 | Pass |
| Parameter | ~$ ping -c 4 10.0.100.11 |  |  |  |
| Response | PING 10.0.100.11 (10.0.100.11) 56(84) bytes of data. 64 bytes from 10.0.100.11: icmp_seq=1 ttl=64 time=4.67 ms 64 bytes from 10.0.100.11: icmp_seq=2 ttl=64 time=2.21 ms 64 bytes from 10.0.100.11: icmp_seq=3 ttl=64 time=2.18 ms 64 bytes from 10.0.100.11: icmp_seq=4 ttl=64 time=2.52 ms --- 10.0.100.11 ping statistics --- 4 packets transmitted, 4 received, 0% packet loss, time 3005ms rtt min/avg/max/mdev = 2.182/2.895/4.668/1.031 ms |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: 64 bytes from 10.0.100.11 contain string: 4 packets transmitted, 4 received, 0% packet loss not contain string: From 10.0.100.10 |  |  |  |
| 39 | Ping from H-1 to H-3 | H-1 | 18:58:26 | Pass |
| Parameter | ~$ ping -c 4 -I Eth1 10.0.200.10 |  |  |  |
| Response | PING 10.0.200.10 (10.0.200.10) from 10.0.100.10 Eth1: 56(84) bytes of data. From 10.0.100.10 icmp_seq=1 Destination Host Unreachable From 10.0.100.10 icmp_seq=2 Destination Host Unreachable From 10.0.100.10 icmp_seq=3 Destination Host Unreachable From 10.0.100.10 icmp_seq=4 Destination Host Unreachable --- 10.0.200.10 ping statistics --- 4 packets transmitted, 0 received, +4 errors, 100% packet loss, time 3063ms pipe 3 |  |  |  |
| Judgement | Success inter-conditional operator : and contain string: From 10.0.100.10 contain string: 4 packets transmitted, 0 received, +4 errors, 100% packet loss not contain string: 64 byte from 10.0.200.10 |  |  |  |
| 40 | Delete Eth1 IPv4 Address on H-1 | H-1 | 18:58:32 | Pass |
| Parameter | ~$ sudo ip address del 10.0.100.10/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 41 | Delete Eth1 IPv4 Address on H-1 | H-1 | 18:58:32 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:01 brd ff:ff:ff:ff:ff:ff inet6 fe80::5054:ff:fe10:201/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and not contain string: DOWN not contain string: inet 10.0.100.10/24 |  |  |  |
| 42 | Delete Eth1 IPv4 Address on H-2 | H-2 | 18:58:32 | Pass |
| Parameter | ~$ sudo ip address del 10.0.100.11/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 43 | Delete Eth1 IPv4 Address on H-2 | H-2 | 18:58:32 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:02 brd ff:ff:ff:ff:ff:ff inet6 fe80::5054:ff:fe10:202/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and not contain string: DOWN not contain string: inet 10.0.100.11/24 |  |  |  |
| 44 | Delete Eth1 IPv4 Address on H-3 | H-3 | 18:58:32 | Pass |
| Parameter | ~$ sudo ip address del 10.0.200.10/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 45 | Delete Eth1 IPv4 Address on H-3 | H-3 | 18:58:32 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:03 brd ff:ff:ff:ff:ff:ff inet 10.0.100.12/24 brd 10.0.100.255 scope global Eth1 valid_lft forever preferred_lft forever inet6 fe80::5054:ff:fe10:203/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and not contain string: DOWN not contain string: inet 10.0.200.10/24 |  |  |  |
| 46 | Delete Eth1 IPv4 Address on H-4 | H-4 | 18:58:32 | Pass |
| Parameter | ~$ sudo ip address del 10.0.200.11/24 dev Eth1 |  |  |  |
| Response | Success |  |  |  |
| 47 | Delete Eth1 IPv4 Address on H-4 | H-4 | 18:58:32 | Pass |
| Parameter | ~$ ip address show Eth1 |  |  |  |
| Response | 3: Eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP group default qlen 1000 link/ether 52:54:00:10:02:04 brd ff:ff:ff:ff:ff:ff inet 10.0.100.13/24 brd 10.0.100.255 scope global Eth1 valid_lft forever preferred_lft forever inet6 fe80::5054:ff:fe10:204/64 scope link valid_lft forever preferred_lft forever |  |  |  |
| Judgement | Success inter-conditional operator : and not contain string: DOWN not contain string: inet 10.0.200.11/24 |  |  |  |